Vision-Based Gesture Recognition for Simulated Robot Control Using MediaPipe and MATLAB

Tejas Y S
Manipal Institute of Technology, Manipal Academy of Higher Education,
Manipal, India
tejas10.mitmpl2025@learner.manipal.edu

K Jumaan 
Manipal Institute of Technology, Manipal Academy of Higher Education,
Manipal, India
jumaan.k@learner.manipal.edu

Narendra Khatri 
Manipal Institute of Technology, Manipal Academy of Higher Education,
Manipal, India
narendra.khatri@manipal.edu

Abstract— This paper presents a lightweight and simulation-driven framework for vision-based gesture recognition and robot control, addressing the gap between gesture classification and validated motion execution in human–robot interaction systems. Unlike conventional approaches that rely on wearable sensors or computationally intensive deep learning models, the proposed system operates entirely on CPU hardware without requiring custom training data. A real-time gesture recognition pipeline is developed using MediaPipe and OpenCV, where five hand gestures—open palm, closed fist, thumbs-up, V-sign, and index pointing—are classified through geometric analysis of 21 hand landmarks. The recognized gestures are mapped to directional commands and integrated with a MATLAB-based differential-drive simulation using standard unicycle kinematics. Experimental evaluation demonstrates high classification performance, achieving 96–100% accuracy across gesture classes with an average latency of 71 ms, satisfying real-time interaction constraints. Furthermore, simulation results confirm that gesture-derived commands produce trajectories consistent with theoretical kinematic predictions, with no observable drift or discontinuity during motion transitions. The key contribution of this work lies in establishing simulation as a critical validation layer between perception and control. By explicitly verifying the gesture-to-motion pipeline, the proposed framework provides a practical, reproducible, and hardware-independent approach for developing reliable vision-based robot control systems.

Keywords— Gesture Recognition, MediaPipe, OpenCV, MATLAB Simulation, Differential-drive Kinematics, Human–Robot Interaction, Computer Vision

# Introduction

Vision-based gesture recognition has become a foundational paradigm for enabling intuitive human–robot interaction (HRI), particularly in applications where conventional control interfaces impose cognitive or physical constraints [1]. By interpreting hand movements through visual sensing, such systems facilitate non-contact, natural communication between humans and robotic agents[2], [3]. Recent advances in computer vision and deep learning have significantly improved the accuracy and responsiveness of gesture recognition systems, with several studies reporting recognition accuracies exceeding 90% under controlled conditions and real-time inference speeds suitable for interactive applications [4], [5].

Nevertheless, the deployment of gesture-driven control systems in practical environments remains non-trivial. Variations in illumination, background complexity, occlusions, and inter-user variability introduce substantial uncertainty in visual perception[6]. For instance, multi-view datasets designed to address occlusion challenges demonstrate that recognition performance can degrade considerably when relying on single-view inputs, thereby necessitating robust feature extraction and aggregation strategies [7]. Furthermore, high-performing deep learning models often require large-scale annotated datasets and computationally intensive training procedures, which may limit their applicability in lightweight or resource-constrained systems[8].

In response to these challenges, recent research has increasingly focused on modular and efficient frameworks that balance performance with computational feasibility. MediaPipe, a widely adopted real-time perception framework, enables precise hand landmark detection with low latency by leveraging optimized machine learning pipelines [8], [9]. Concurrently, MATLAB provides a versatile environment for system modeling, simulation, and control design, allowing rapid prototyping and validation of robotic systems [9]. Therefore, integrating MediaPipe-based hand tracking with MATLAB-driven simulation offers a practical and scalable approach for developing gesture-controlled robotic systems [10].

Moreover, gesture recognition pipelines typically involve sequential stages, including hand detection, key point extraction, feature representation, and classification. While deep neural networks-particularly convolutional neural networks (CNNs) and attention-based architectures-have demonstrated superior performance, lightweight approaches remain essential for real-time deployment [11], [12], [13], [14]. For example, monocular vision-based systems have achieved gesture recognition accuracies of up to 94.11% with average response times of approximately 0.615 seconds, highlighting the feasibility of non-contact interaction in real-world applications.

Accordingly, this study proposes a vision-based gesture recognition framework that integrates MediaPipe for robust hand landmark extraction with MATLAB for simulated robot control. The objective is to develop a system that achieves reliable gesture interpretation while maintaining computational efficiency and ease of implementation [15], [16]. By emphasizing simulation-driven validation, the proposed approach also provides a controlled environment for evaluating gesture-to-action mappings before deployment in physical robotic platforms.

## Background and Prior Work

The domain of vision-based gesture recognition for HRI has witnessed substantial progress, driven by advancements in deep learning, sensor technologies, and real-time computing. A comprehensive review of recent literature indicates that convolutional neural networks (CNNs) and recurrent neural networks (RNNs) remain dominant in gesture recognition tasks, while emerging paradigms such as transformers, graph neural networks, and vision-language models are gaining traction for capturing complex spatial and temporal dependencies. Despite these developments, several limitations persist, including the scarcity of large-scale HRI-specific datasets, limited generalization across diverse environments, and insufficient benchmarking for real-time performance [17], [18], [19].

In industrial and collaborative robotics, gesture recognition has been explored as a means to enhance operational flexibility and reduce reliance on physical interfaces. Multi-view gesture recognition frameworks have demonstrated improved robustness against occlusions and viewpoint variations by aggregating information from multiple camera perspectives. Experimental evaluations on datasets such as MuViH show that multi-view aggregation can significantly enhance recognition accuracy compared to single-view approaches, particularly in complex industrial scenarios[7], [10], [20], [21].

Similarly, vision-based control systems for robotic manipulators have been developed to map human gestures directly into robot actions. A three-stage pipeline comprising gesture detection, tracking, and control has been shown to enable intuitive manipulation of robotic arms, with adaptive inverse kinematics ensuring smooth trajectory generation. These systems reduce operator training requirements and improve usability, although challenges related to latency and real-time synchronization remain [22], [23], [24], [25].

To address temporal dynamics in gesture recognition, spatio-temporal modeling techniques have been proposed. Transformer-based architectures, for instance, capture motion continuity by analyzing sequential image frames, thereby improving robustness in dynamic and visually challenging environments such as underwater settings. Empirical results indicate that incorporating temporal information can significantly enhance recognition accuracy and reduce misclassification caused by transient visual disturbances [26].

In parallel, efforts have been made to design lightweight and efficient models suitable for real-time deployment. Networks such as dual-stage architectures with attention mechanisms optimize feature fusion by selectively integrating local and global information, thereby improving both segmentation and recognition performance while maintaining low computational overhead. Experimental studies report reduced parameter sizes alongside competitive accuracy, demonstrating the feasibility of deploying such models on resource-constrained platforms [27], [28].

Non-contact interaction methods have also gained prominence, particularly in assistive and rehabilitation robotics. Vision-based systems utilizing monocular cameras and key point detection techniques have achieved gesture-controlled operation with accuracies exceeding 94% and sub-second response times, addressing issues of comfort, reliability, and ease of use associated with contact-based interfaces. These approaches underscore the potential of vision-based HRI in applications requiring user-friendly and unobtrusive interaction mechanisms [17], [29].

Additionally, classical computer vision techniques continue to play a role in gesture recognition, particularly in scenarios where deep learning resources are limited. Methods based on histogram of oriented gradients (HOG) features combined with support vector machine (SVM) classifiers have demonstrated reliable performance in controlled environments, although they are generally less robust to background variability and illumination changes [30], [31].

Furthermore, attention-based CNN architectures and densely connected networks have been proposed to enhance feature representation, particularly for finger-level gesture recognition. These models emphasize critical regions of the hand, improving classification accuracy in complex scenes. Experimental evaluations on benchmark datasets confirm their superiority over conventional architectures in terms of mean accuracy and generalization capability[32], [33]. Table I presents the four generations of gesture recognition research in the domain.

Despite these advancements, a clear gap remains in the development of integrated, simulation-oriented frameworks that combine efficient hand tracking with intuitive robot control. Many existing approaches rely on complex training pipelines or specialized hardware, limiting their accessibility and scalability. Therefore, the present work addresses this gap by proposing a MediaPipe-based gesture recognition system integrated with MATLAB simulation, offering a lightweight, modular, and practical solution for vision-based robot control.

Four generations of gesture recognition research.

| Era | Period | Technology | Key Limitation |
| --- | --- | --- | --- |
| Sensor gloves | 2010–2015 | IMU / flex sensors | Wearable hardware |
| Classical vision | 2015–2019 | HSV, contour | Light-sensitive |
| Deep learning | 2018–2021 | CNN, TFLite | GPU / large dataset |
| Lightweight AI | 2020–now | MediaPipe | Limited customization |

## What This Work Contributes

Most existing studies emphasize either gesture recognition accuracy or direct robotic implementation, often neglecting the intermediate validation of control logic. Although high classification accuracies (typically above 90%) are frequently reported, they do not inherently ensure correct or stable robot motion.

This work addresses that gap by integrating gesture recognition with a simulation-driven validation framework. A MediaPipe-based system is used to reliably classify five control gestures in real time on a standard laptop, ensuring computational efficiency. The proposed system was developed considering practical real-world conditions, including variations in illumination, background complexity, and hand orientation. The MediaPipe-based framework achieved stable real-time gesture recognition using a standard laptop webcam without specialized hardware. Moreover, the geometric feature-based approach ensured low computational overhead and reliable responsiveness, making the system suitable for real-time human–robot interaction applications.

In addition, geometric methods were selected due to their low computational overhead, robustness to illumination and background variations, and suitability for real-time hand landmark analysis without requiring extensive training data. These recognized gestures are then mapped to a differential-drive kinematic model in MATLAB, enabling the generation of expected robot trajectories.

By explicitly linking perception to motion through simulation, the proposed approach verifies that recognized gestures translate into correct robotic behavior. This establishes simulation as a critical validation layer between “gesture recognized” and “robot moves correctly,” thereby providing a practical and computationally efficient pathway for system verification prior to real-world deployment.

# PROBLEM DEFINITION

The problem can be framed in terms of usability, computational feasibility, and system validation. Conventional robot control interfaces-such as joysticks, mobile applications, or Bluetooth remotes-remain widely used; however, they impose cognitive load on the operator and restrict interaction to device-centric control rather than task-centric engagement. As a result, these interfaces are neither intuitive nor hands-free, limiting their effectiveness in dynamic human-robot interaction scenarios.

Gesture-based control has been explored as a more natural alternative; however, existing solutions often rely on wearable sensing devices or computationally intensive deep learning models. Wearable systems, such as sensor gloves, introduce additional hardware dependencies and reduce user convenience. In contrast, vision-based deep learning approaches typically require large, annotated datasets and GPU-enabled training pipelines, creating a high barrier to entry for lightweight or simulation-focused implementations.

Accordingly, the core problem addressed in this work is the development of a gesture recognition system that operates efficiently on standard CPU hardware without requiring custom model training, while still providing reliable and meaningful control outputs. Furthermore, beyond recognition, the system must enable validation of control logic through a simulation environment, ensuring that interpreted gestures translate into correct robotic motion. This requirement is critical, as prior work highlights the lack of consistent evaluation frameworks linking perception outputs to control behavior.

To address these challenges, the proposed approach adopts a design that integrates real-time vision-based gesture recognition with simulation-driven validation. Specifically, the system is designed to (i) operate using pre-trained models without additional training overhead, (ii) support a defined set of directional gestures suitable for robot navigation, and (iii) generate outputs compatible with a differential-drive kinematic model for trajectory verification in MATLAB.

Table II summarizes the identified limitations in existing approaches and the corresponding design responses adopted in this work.

Identified limitations and corresponding design responses.

| Limitation | Effect | Approach |
| --- | --- | --- |
| Physical controllers | Not hands-free | Camera + gesture |
| Sensor gloves | Extra hardware | MediaPipe (no glove) |
| GPU / dataset | High barrier | CPU-only, pre-trained |
| No validation path | Logic unverifiable | MATLAB simulation |

# METHODOLOGY

## Software Environment

The gesture recognition side runs entirely in Python 3.9. OpenCV 4.x handles camera frame acquisition, color-space conversion, and the display overlay. MediaPipe 0.10 provides the hand-landmark model. Testing was done on a standard laptop-no embedded board, no special hardware. The MATLAB simulation runs in R2023b using a custom differential-drive kinematic script described in Section III-C.

## Experimental Setup and Testing Conditions

The experimental validation was conducted under controlled indoor laboratory conditions using a standard laptop equipped with an Intel Core i5 processor, 8 GB RAM, and an integrated webcam operating at 30 fps with a frame resolution of 1280 × 720 pixels. The gesture recognition framework was executed in real time using Python 3.9, OpenCV 4.x, and MediaPipe 0.10, while the robot motion validation was performed through MATLAB R2023b simulations. During testing, the user was positioned approximately 0.5–1.2 m from the camera to ensure reliable hand landmark acquisition. Ambient lighting conditions were maintained within normal indoor illumination levels (~300–500 lux) to minimize landmark detection instability caused by shadows or overexposure.

Five predefined gesture classes, namely Forward, Reverse, Turn Left, Turn Right, and Stop, were evaluated through repeated trials under varying hand orientations and moderate background variations. Each gesture was tested across 50 independent trials to evaluate classification consistency, command stability, and response latency. The average processing latency, including landmark extraction and gesture classification, was measured in real time to assess suitability for interactive robotic control. Furthermore, the MATLAB-based differential-drive simulation validated whether the recognized gestures generated the expected robot trajectories and motion responses under the defined wheel-velocity constraints.

## Gesture Recognition Pipeline

Each video frame from the webcam is passed to MediaPipe’s HandLandmarker. It returns 21 (x, y, z) landmark coordinates, normalized to frame dimensions. From those, gesture classification uses three geometric checks — no pixel matching, no template comparison, just landmark geometry.

First, finger extension. For each finger i, the landmark tip is compared to the proximal interphalangeal (PIP) joint in the y-axis (lower y = higher in the frame = extended):

extendedi= 1  if  LMtipy<  LMpipy

else  0   ……… (1)

Second, fingertip spread. To separate a closed fist from an open palm, the average distance from each fingertip to the wrist landmark is computed:

D = (1/N) × Σ ||LM_tip_i - LM_wrist||   ……… (2)

Third, thumb direction. The thumb tip is compared to the thumb metacarpophalangeal (MCP) joint to detect a thumbs-up:

thumbup= 1  if  LMthumbtipy<  LMthumbmcpy

else  0   ……… (3)

Table III presents five gesture classes defined from combinations of these checks:

Gesture classes and geometric decision rules.

| Gesture | Command | Rule |
| --- | --- | --- |
| Open palm | Forward | All 5 fingers extended (eq. 1) |
| Closed fist | Stop | D < threshold (eq. 2) |
| Thumbs-up | Turn Right | thumb_up=1, others curled |
| Peace / V-sign | Turn Left | Index+middle ext., rest curled |
| Index point | Reverse | Index extended only |

## Differential-Drive Kinematic Model

The MATLAB simulation models a generic differential-drive robot — the same kinematic architecture used by most small, wheeled platforms. Given left and right wheel velocities vₗ and vᵣ, the robot’s linear and angular velocities are:

v = (vr + vl) / 2   ……… (4)

ω = (vr - vl) / L   ……… (5)

where L = 0.14 m is the assumed wheelbase — a typical value for small educational robots. Pose (x, y, θ) updates at each timestep Δt = 0.1 s using the standard Euler integration:

x(t+1) = x(t) + v × cos(θ) × Δt   ……… (6)

y(t+1) = y(t) + v × sin(θ) × Δt   ……… (7)

θ(t+1) = θ(t) + ω × Δt   ……… (8)

Each gesture command maps to a specific wheel-velocity pair. Forward sets vᵣ = vₗ = 0.3 m/s (straight line). Turn Right sets vᵣ = 0.3, vₗ = −0.3 m/s (point turn). Turn Left is the mirror image. Stop sets both to zero. The turning radius R for any command follows from equations (4) and (5):

R = v / ω = L(vr + vl) / [2(vr - vl)]   ……… (9)

For a point turn (vᵣ = −vₗ), R = 0. For straight motion, R → ∞. The gesture-to-wheel velocity mapping adopted in the simulation framework is presented in Table IV.

Gesture-to-wheel-velocity mapping used in the simulation.

| Command | vₗ (m/s) | vᵣ (m/s) |
| --- | --- | --- |
| Forward | 0.3 | 0.3 |
| Reverse | −0.3 | −0.3 |
| Turn Right | −0.3 | 0.3 |
| Turn Left | 0.3 | −0.3 |
| Stop | 0.0 | 0.0 |

# RESULTS AND DISCUSSION

## Gesture Recognition — Live Results

Figures 1–4 present the real-time behavior of the MediaPipe-based gesture recognition module during live webcam testing. The overlaid hand landmarks and corresponding labels indicate both the detected gesture and its mapped control command. Under standard indoor illumination and without any task-specific retraining, the system demonstrated stable and repeatable recognition across all four gesture classes. This is notable, as several vision-based systems reported in the literature require either controlled environments or dataset-specific tuning to maintain comparable stability.

From a quantitative perspective, the system achieved classification accuracies of 98% (open palm), 100% (closed fist), 96% (thumbs-up), and 98% (peace sign) over 50 trials per gesture. These results are consistent with, and in some cases comparable to, deep learning-based approaches that report accuracies above 90% but often rely on large, annotated datasets and GPU-based training pipelines. In contrast, the present system operates entirely on pre-trained models with CPU-only execution, thereby reducing computational overhead while maintaining high recognition performance. The system maintained stable real-time performance using a standard laptop webcam operating at conventional resolution and frame rates, indicating that the landmark-based framework does not require high-end imaging hardware for reliable gesture recognition.

Latency measurements further reinforce the system’s suitability for real-time interaction. The observed response time, ranging between 65 ms and 78 ms with an average of 71 ms, falls well below the commonly accepted 100 ms threshold for responsive HRI systems. The classification accuracy and latency performance across 50 experimental trials for each gesture are summarized in Table V. This performance is comparable to lightweight CNN-based models and significantly more efficient than transformer-based or spatio-temporal architectures, which, although more expressive, often introduce additional computational delay due to sequence modeling.

A critical observation relates to robustness under practical constraints. The proposed framework also mitigates the influence of lighting variations and background noise through MediaPipe-based hand landmark extraction, which utilizes geometric hand features instead of raw pixel information. This improves robustness and maintains stable real-time gesture recognition under typical indoor operating conditions. While prior works highlight challenges such as occlusion, viewpoint variation, and background complexity, the present system exhibited only one consistent failure mode—partial hand occlusion near the frame boundaries. Importantly, this limitation was easily mitigated through simple user positioning, suggesting that the system maintains operational robustness without requiring multi-view setups or additional sensing modalities. Compared to multi-camera or multi-modal approaches proposed in existing studies, which improve accuracy at the cost of system complexity, the current approach achieves a favorable balance between simplicity and performance.

Classification accuracy and latency across 50 trials per gesture.

| Gesture | Trials | Correct | Accuracy |
| --- | --- | --- | --- |
| Open palm | 50 | 49 | 98% |
| Closed fist | 50 | 50 | 100% |
| Thumbs-up | 50 | 48 | 96% |
| Peace sign | 50 | 49 | 98% |
| Avg. latency | — | — | 71 ms |

Overall, the results indicate that reliable gesture recognition can be achieved without custom training or high-end hardware, provided that efficient landmark-based representations are employed. This positions the proposed system as a practical alternative to more resource-intensive methods, particularly for rapid prototyping and simulation-driven validation.

Fig. 1: OPEN → FORWARD

Fig. 2: FIST → STOP.

Fig. 3: THUMBS_UP → TURN_RIGHT.

Fig. 4: PEACE → TURN_LEFT.

## MATLAB Simulation Results

The MATLAB-based simulation serves as a critical validation layer, bridging the gap between gesture recognition and robot motion execution. Figure 5 illustrates the temporal sequence of commands for a representative test case, where discrete gesture inputs are mapped to motion primitives such as forward movement, turning, and stopping. The transitions between states are sharp and consistent, with no observable latency between gesture detection and control update. This contrasts with several existing implementations where delays in perception-to-control pipelines introduce instability or lag in robot response.

Fig. 5. MATLAB command-sequence plot. Each step corresponds to a detected gesture.

Fig. 6. Top-view trajectory from the MATLAB simulation. Labels show active gesture command at each segment.

Figure 6 presents the resulting top-view trajectory generated by the differential-drive kinematic model. The trajectory follows an S-shaped path corresponding to the commanded sequence: linear motion, left turn, linear continuation, left and right turn. Importantly, the observed motion aligns precisely with the theoretical predictions of the kinematic equations governing differential-drive systems. No unintended drift, overshoot, or discontinuity was detected, even at transition points between motion states.

In comparison with prior work, many gesture-controlled robotic systems validate performance primarily through classification metrics or hardware demonstrations, often without explicitly verifying whether the generated motion adheres to expected kinematic behavior. While some studies implement real-time control of robotic manipulators or mobile platforms, they frequently lack a structured simulation framework that isolates and evaluates the correctness of control logic. As a result, it becomes difficult to distinguish whether performance limitations arise from perception errors, control inaccuracies, or hardware constraints.

The present work addresses this limitation by introducing simulation as an explicit validation stage. By mapping gesture outputs to wheel velocity commands and evaluating the resulting trajectories within a controlled MATLAB environment, the system enables precise verification of motion behavior independent of hardware noise. This approach aligns with emerging research directions that emphasize the need for systematic benchmarking of end-to-end HRI pipelines rather than isolated component evaluation.

Furthermore, compared to hardware-centric implementations, the simulation-driven approach offers several advantages, including reproducibility, ease of parameter tuning, and the ability to test edge cases without risk. It also provides a scalable pathway for extending the system to more complex control strategies, such as trajectory planning or multi-agent coordination, before physical deployment.

In summary, the simulation results confirm that the proposed system not only achieves accurate gesture recognition but also ensures that these gestures translate into physically consistent and predictable robot motion. By explicitly validating the gesture-to-motion pipeline, this work advances beyond conventional approaches and establishes a more rigorous framework for developing reliable vision-based robot control systems.

# CONCLUSION

This study presented a vision-based gesture recognition framework integrated with simulation-driven validation for mobile robot control. The proposed approach successfully addresses key limitations in existing systems, namely dependence on specialized hardware, requirement for large training datasets, and lack of structured validation between perception and control. By leveraging MediaPipe for real-time hand landmark detection and MATLAB for kinematic simulation, the system achieves both computational efficiency and functional reliability. Experimental results demonstrate that the gesture recognition module attains high accuracy (96–100%) with low latency (≈71 ms), ensuring responsiveness suitable for real-time HRI applications. More importantly, the simulation results confirm that recognized gestures are consistently translated into correct and predictable robot motion, validating the integrity of the end-to-end pipeline. In contrast to prior work that evaluates components in isolation, this study emphasizes the importance of simulation as a validation layer, enabling systematic verification of control logic before hardware deployment. The proposed framework thus offers a scalable and accessible foundation for future research in gesture-driven robotics, with potential extensions toward real-world implementation, multi-modal interaction, and adaptive control strategies.

##### References

[1]	T. Wang, P. Zheng, S. Li, and L. Wang, “Multimodal Human–Robot Interaction for Human-Centric Smart Manufacturing: A Survey,” Advanced Intelligent Systems, vol. 6, no. 3, p. 2300359, Mar. 2024, doi: 10.1002/AISY.202300359;WGROUP:STRING:PUBLICATION.

[2]	J. Bernard, M. Hutter, M. Zeppelzauer, D. Fellner, and M. Sedlmair, “Comparing Visual-Interactive Labeling with Active Learning: An Experimental Study,” IEEE Trans. Vis. Comput. Graph., vol. 24, no. 1, pp. 298–308, 2018, doi: 10.1109/TVCG.2017.2744818.

[3]	J. Bernard, M. Zeppelzauer, M. Sedlmair, and W. Aigner, “VIAL: a unified process for visual interactive labeling,” Vis. Comput., vol. 34, no. 9, pp. 1189–1207, 2018, doi: 10.1007/s00371-018-1500-3.

[4]	J. Qi, L. Ma, Z. Cui, and Y. Yu, “Computer vision-based hand gesture recognition for human-robot interaction: a review,” Complex and Intelligent Systems, vol. 10, no. 1, pp. 1581–1606, Feb. 2024, doi: 10.1007/S40747-023-01173-6/TABLES/6.

[5]	E. Low-Cost et al., “Low-Cost Computer-Vision-Based Embedded Systems for UAVs,” Robotics 2023, Vol. 12, Page 145, vol. 12, no. 6, p. 145, Oct. 2023, doi: 10.3390/ROBOTICS12060145.

[6]	N. X. Tran et al., “Wireless Data Glove for Gesture-Based Robotic Control,” Lecture Notes in Computer Science (including subseries Lecture Notes in Artificial Intelligence and Lecture Notes in Bioinformatics), vol. 5611 LNCS, no. PART 2, pp. 271–280, 2009, doi: 10.1007/978-3-642-02577-8_30.

[7]	C. Hubert, N. Odic, M. Noel, S. Gharib, S. H. H. Zargarbashi, and L. Séoud, “MuViH: Multi-View Hand gesture dataset and recognition pipeline for human–robot interaction in a collaborative robotic finishing platform,” Robot. Comput. Integr. Manuf., vol. 94, Aug. 2025, doi: 10.1016/j.rcim.2025.102957.

[8]	Y. YASEEN, O.-J. Kwon, J. Kim, F. Ullah, J. Lee, and S. Jamil, “Comparative Analysis of Hand Gesture Datasets for Drone Control Using Mediapipe,” 2024, doi: 10.2139/SSRN.4876970.

[9]	C. Lugaresi et al., “MediaPipe: A Framework for Building Perception Pipelines,” Jun. 2019, Accessed: Apr. 23, 2026. [Online]. Available: https://arxiv.org/pdf/1906.08172

[10]	C. Scoccia, G. Menchi, M. Ciccarelli, M. Forlini, and A. Papetti, “Adaptive Real-Time Gesture Recognition in a Dynamic Scenario for Human-Robot Collaborative Applications,” Mechanisms and Machine Science, vol. 122 MMS, pp. 637–644, 2022, doi: 10.1007/978-3-031-10776-4_73.

[11]	Q. Li and R. Langari, “Myoelectric human computer interaction using CNN-LSTM neural network for dynamic hand gesture recognition,” Journal of Intelligent and Fuzzy Systems, vol. 44, no. 3, pp. 4207–4221, Nov. 2023, doi: 10.3233/JIFS-222985;PAGE:STRING:ARTICLE/CHAPTER.

[12]	K. Gupta, A. Singh, S. R. Yeduri, M. B. Srinivas, and L. R. Cenkeramaddi, “Hand gestures recognition using edge computing system based on vision transformer and lightweight CNN,” Journal of Ambient Intelligence and Humanized Computing 2022 14:3, vol. 14, no. 3, pp. 2601–2615, Dec. 2022, doi: 10.1007/S12652-022-04506-4.

[13]	D. K. Singh, “3D-CNN based Dynamic Gesture Recognition for Indian Sign Language Modeling,” Procedia Comput. Sci., vol. 189, pp. 76–83, Jan. 2021, doi: 10.1016/J.PROCS.2021.05.071.

[14]	J. O. Pinzón-Arenas, R. Jiménez-Moreno, and A. Rubiano, “Percentage estimation of muscular activity of the forearm by means of EMG signals based on the gesture recognized using CNN,” Sens. Biosensing Res., vol. 29, p. 100353, Aug. 2020, doi: 10.1016/J.SBSR.2020.100353.

[15]	J. Bora, S. Dehingia, A. Boruah, A. A. Chetia, and D. Gogoi, “Real-time Assamese Sign Language Recognition using MediaPipe and Deep Learning,” Procedia Comput. Sci., vol. 218, pp. 1384–1393, Jan. 2023, doi: 10.1016/J.PROCS.2023.01.117.

[16]	W. Cao, P. Lu, and W. Cao, “Multimodal Gesture Recognition with Spatio-Temporal Features Fusion Based on YOLOv5 and MediaPipe,” https://doi.org/10.1142/S0218001424550073, vol. 38, no. 8, Jun. 2024, doi: 10.1142/S0218001424550073.

[17]	S. Qiu, Y. Liu, C. Wang, J. Zhang, W. Xian, and C. Chen, “Vision-Based Gesture-Driven Control Method for Rehabilitation Lower Extremity Exoskeleton Robots,” IEEE Sens. J., 2025, doi: 10.1109/JSEN.2025.3620836.

[18]	C. M. De Melo, B. Rothrock, P. Gurram, O. Ulutan, and B. S. Manjunath, “Vision-based gesture recognition in human-robot teams using synthetic data,” IEEE International Conference on Intelligent Robots and Systems, pp. 10278–10284, Oct. 2020, doi: 10.1109/IROS45743.2020.9340728.

[19]	J. Qi, L. Ma, Z. Cui, and Y. Yu, “Computer vision-based hand gesture recognition for human-robot interaction: a review,” Complex and Intelligent Systems, vol. 10, no. 1, pp. 1581–1606, Feb. 2024, doi: 10.1007/S40747-023-01173-6/TABLES/6.

[20]	P. Barattini, C. Morand, and N. M. Robertson, “A proposed gesture set for the control of industrial collaborative robots,” Proceedings - IEEE International Workshop on Robot and Human Interactive Communication, pp. 132–137, 2012, doi: 10.1109/ROMAN.2012.6343743.

[21]	P. Segura, O. Lobato-Calleros, A. Ramírez-Serrano, and I. Soria, “Human-robot collaborative systems: Structural components for current manufacturing applications,” Advances in Industrial and Manufacturing Engineering, vol. 3, p. 100060, Nov. 2021, doi: 10.1016/j.aime.2021.100060.

[22]	Y. M. R. da Silva et al., “Computer Vision Based Path Following for Autonomous Unmanned Aerial Systems in Unburied Pipeline Onshore Inspection,” Drones 2022, Vol. 6, Page 410, vol. 6, no. 12, p. 410, 2022, doi: 10.3390/DRONES6120410.

[23]	Q. Liu and Q. Cong, “Kinematic and dynamic control model of wheeled mobile robot under internet of things and neural network,” The Journal of Supercomputing 2021 78:6, vol. 78, no. 6, pp. 8678–8707, Jan. 2022, doi: 10.1007/S11227-021-04160-1.

[24]	O. Alparslan and O. Cetin, “Real-Time Indoor Path Planning Using Object Detection for Autonomous Flying Robots,” Intelligent Automation & Soft Computing, vol. 36, no. 3, pp. 3355–3370, 2023, doi: 10.32604/IASC.2023.035689.

[25]	N. Fallahinia and S. A. Mascaro, “Real-Time Tactile Grasp Force Sensing Using Fingernail Imaging via Deep Neural Networks,” IEEE Robot. Autom. Lett., vol. 7, no. 3, pp. 6558–6565, Jul. 2022, doi: 10.1109/LRA.2022.3173751.

[26]	Z.-H. Zhang et al., “Action Recognition for Underwater Gesture Communication in Human Diver and Robot Teaming,” IEEE International Conference on Intelligent Robots and Systems , pp. 16939–16946, Nov. 2025, doi: 10.1109/IROS60139.2025.11247289.

[27]	R. Girshick, J. Donahue, T. Darrell, and J. Malik, “Rich feature hierarchies for accurate object detection and semantic segmentation,” Proceedings of the IEEE Computer Society Conference on Computer Vision and Pattern Recognition, pp. 580–587, Nov. 2013, doi: 10.48550/arxiv.1311.2524.

[28]	R. Girshick, J. Donahue, T. Darrell, and J. Malik, “Rich feature hierarchies for accurate object detection and semantic segmentation,” Proceedings of the IEEE Computer Society Conference on Computer Vision and Pattern Recognition, pp. 580–587, 2014, doi: 10.1109/CVPR.2014.81.

[29]	A. Saikia and S. Paul, “EEG signal processing and its classification for rehabilitation device control,” Application of Biomedical Engineering in Neuroscience, pp. 173–196, Nov. 2019, doi: 10.1007/978-981-13-7142-4_9/FIGURES/8.

[30]	V. Adithya and R. Rajesh, “A Deep Convolutional Neural Network Approach for Static Hand Gesture Recognition,” Procedia Comput. Sci., vol. 171, pp. 2353–2361, Jan. 2020, doi: 10.1016/J.PROCS.2020.04.255.

[31]	J. P. Sahoo, S. P. Sahoo, S. Ari, and S. K. Patra, “Hand Gesture Recognition Using Densely Connected Deep Residual Network and Channel Attention Module for Mobile Robot Control,” IEEE Trans. Instrum. Meas., vol. 72, 2023, doi: 10.1109/TIM.2023.3246488.

[32]	R. Jalayer, M. Jalayer, C. Orsenigo, and M. Tomizuka, “A review on deep learning for vision-based hand detection, hand segmentation and hand gesture recognition in human–robot interaction,” Robot. Comput. Integr. Manuf., vol. 97, Feb. 2026, doi: 10.1016/j.rcim.2025.103110.

[33]	H. Sümbül, “A Novel Mems and Flex Sensor-Based Hand Gesture Recognition and Regenerating System Using Deep Learning Model,” IEEE Access, vol. 12, pp. 133685–133693, 2024, doi: 10.1109/ACCESS.2024.3448232.